Town of Richmond

Planning & Zoning Office

203 Bridge Street, P.O. Box 285

Richmond, VT 05477

www.richmondvt.gov

Richmond Development Review Board

Revised Staff Notes on Item Two

SB2026-001

APPLICANT: Trunk Beach, LLC (c/o Nate Malley).

RE: Final Subdivision, SB2026-001.

HEARING DATES: June 10, 2026, and July 8, 2026.

LOCATION: 30 Browns Court.

PARCEL ID: BR0030.

EXISTING ZONING: Village Residential Neighborhoods North (VRNN) Zoning District.

- FLOOD HAZARD OVERLAY DISTRICT: No.

- SHORELINE PROTECTION OVERLAY DISTRICT: No.

PROJECT DESCRIPTION:

Applicant seeks final approval to subdivide 30 Browns Court, an existing 0.47-acre parcel into two (2) parcels of 0.25 acres (Lot 1) and 0.22 acres (Lot 2). Lot 1 will get a newly relocated 12-foot-wide driveway. No additional changes to Lot 1 are proposed. Lot 2 will get a six (6)-bedroom duplex (three bedrooms per unit) and a shared 30-foot-wide driveway served by municipal water and sewer lines. Most of the existing tree line boarding the park will be cut back; in its place will go seven (7) “green giant” arborvitae trees.

PROCEDURAL HISTORY:

- Preliminary Approval granted by the DRB on March 27, 2026.

- Pre-submission meetings held with Staff in April and May 2026.

- CUR Application finalized and accepted for DRB review on May 20, 2026.

- Public notice forwarded to DRB on May 21, 2026, and June 23, 2026.

- Public notice mailed to Applicant and Adjoining Property Owners on May 27, 2026.

- Public notice posted to Town’s website and at three (3) public locations on May 27, 2026, and June 23, 2026.

- Public notice placed in Seven Days, the publication of general circulation, on May 27, 2026, and July 1, 2026.

RELATED SUBMISSIONS:

- [2.1] Application, Narrative, WW Permit, Draft Warranty Deed, Preliminary WW Allocation, and Abutters.

- [2.2] Plat.

- [2.3] Amended Site plan.

- [2.4] Amended Landscaping plan.

- [2.5] Amended Construction plans.

- [2.6] Architecturals.

- [2.7] Engineering Certification.

- [2.8] Written Public Comment.

- [2.9] HydroCAD Results.

- [2.10] Stormwater Analysis of HydroCAD.

AUTHORITIES:

- The division of one lot into two or more lots requires subdivision approval before the DRB.

  - Procedurally, the DRB must first approve an application for Preliminary Subdivision before it can approve a Final Subdivision.

    - The DRB may generally vote to waive or modify, subject to appropriate conditions, any application requirement except those for public roads or infrastructure.

    - The DRB may attach all reasonable conditions and safeguards necessary to implement the purposes of relevant regulations, bylaws, or standards, and to mitigate any undue adverse effects associated with the subdivision.

  - No one lot shall be subdivided until an Applicant has (i) obtained approval of the Final Subdivision from the DRB and (ii) recorded a copy of the approved Final Subdivision Plat with the Town Clerk.

    - Applicant has 180 days from the date of final subdivision approval to record a copy of the Final Subdivision Plat with the Town Clerk; otherwise, the approval shall expire.

- DRB approvals shall expire 24 months from the date of final approval unless a valid Zoning Permit has been obtained or the DRB determines at a warned public hearing to grant an extension.

STANDARD OF REVIEW:

For final subdivision approval, Applicant must satisfy the three (3) following requirements.

- Submit all required information for the (1.1) final subdivision plat, and the (1.2) final subdivision plan.

- Certify that field makers are located on-site and will be maintained there until a final decision is made by the DRB on the application.

- When and where applicable, address all required improvements and design standards limited to (3.1) roads, (3.2) lot layout, (3.3) pedestrian and bicycle access, (3.4) outdoor lighting, (3.5) landscaping, (3.6) drainage and fill, (3.7) water supply and wastewater disposal systems, (3.8) utilities, and (3.9) phasing.

The DRB may also require Applicant to provide additional information related to twelve general planning standards. RSR §§ 500(1)-(12). Some general planning standards include, but are not limited to, general health and safety standards, undue adverse impacts, control of runoff and erosion during and after construction, and site suitability.

WRITTEN PUBLIC COMMENT

On June 22, 2026, neighbor and parks committee member Fran Thomas submitted written public comments on the proposed subdivision. Thomas effectively raised three items of concern, pertaining to landscaping, portalet screening, and stormwater. The DRB should consider Thomas’ comments and elicit any further testimony on the topics.

FINAL PLAT AND PLAN REVIEW (RSR §§ 300-320)

No immediate concerns from Saff. Notwithstanding the waivers requested for the specific drainage requirements, Staff agrees that Applicant meet all required submission elements for Final Plat and Plan Review.

- Applicant satisfied all copy requirements by submitting three copies of the Final Plat and Plan.

- Applicant satisfied all Final Plat requirements listed under RSR § 420.2.

- Applicant satisfied all Final Plan requirements listed under RSR § 420.3.

REQUIRED IMPROVEMENTS AND DESIGN STANDARDS: LOT LAYOUT, OUTDOOR LIGHTING, LANDSCAPING, DRAINAGE, and EROSION CONTROL (RSR §§ 600-680)

Staff believes that Applicant has adequately demonstrated compliance with prior conditions for lot layout, outdoor lighting, landscaping, and drainage. However, the DRB reserves the right to review these changed features and impose any further conditions as they see fit. The DRB should elicit additional information and testimony on the proposed drainage features.

- Lot Layout (RSR § 610)

Applicant moved the boundary line and adjusted the lot sizes to avoid violations of the ten (10) foot side setback for principal structures within the Village Residential Neighborhoods North Zoning District. The basement bulkhead and stairs to the back deck are now depicted as falling outside of the required setbacks.

- Drainage & Erosion Control (RSR § 650)

As required by the Subdivision Regulations, an Applicant “shall provide a design for a drainage system which will remove any water and storm run-off which may traverse the sight.” Moreover, “[d]rainage facilities shall accommodate all existing surface water and run-off, plus run-off anticipated from the proposed development. These drainage systems shall be designed to accommodate water from a twenty-five (25) year storm. And, as to erosion control plans, an Applicant “shall install permanent vegetation and erosion/runoff control measures as soon as practical.”

Applicant has proposed a dry pond and associated outlet pipe to handle drainage. In support of the plans, Applicant submitted HydroCAD results and written analysis for the design and construction of adequate drainage systems. Therein, Applicant certified that the post-routed peak flow (0.60 cfs) is less than the existing peak flow (0.72 cfs) from the newly created lot. In their analysis, the Applicant focused on pre and post flows for Lot 2 only. While Lot 1 will remain unchanged except for a relocated driveway, the DRB may wish for Applicant to explain why Lot 1 was not included in the HydroCAD modeling and discuss how the proposed drainage plans satisfy the 25-year storm standard.

- Outdoor Lighting (RSR § 630; RZR § 4.10)

Unbeknownst to the Zoning Administrator at the time, Applicant correctly pointed out that outdoor and exterior lighting associated with normal and customary residential activities is exempt from heightened lighting standards. As such, Staff agree that Applicant’s residential subdivision falls under this exemption. By claiming and utilizing the exemption, Staff believe that the Applicant has provided an adequate outdoor lighting plan that complies with both the Subdivision and Zoning regulations.

- Landscaping (RSR § 640)

The DRB required Applicant to provide a planning plan that follows provisions for new trees or shrubs, and landscaping.

Applicant has proposed cutting back most of the existing tree line boarding the park and planting seven (7) “green giant” arborvitae trees (Thuja plicata x Thuja standishii). The green giant trees will restore a landscaping and screening buffer between the park and the proposed duplex.

According to the Vermont Tree Selection Guide, the green giant trees are native to Vermont. Considering that the green giant trees are apparently native to Vermont, the requirement for indigenous plantings appears to be achieved.

GENERAL PLANNING STANARDS (RSR § 500)

No immediate concerns from Staff. The DRB may consider requiring the Applicant to submit additional information related to “[w]hether the proposed subdivision includes adequate provision for the control of runoff and erosion during and after construction.” This could be used as further justification for the need of adequate drainage, stormwater runoff, and erosion control. The DRB may also consider asking about plans for the two (2) 8’ x 10’ back decks that are depicted in the Architecturals but not on the Plat or Plan.

STAFF COMMENTS AND RECOMMENDATIONS:

- Comments: The DRB should focus on the adequacy of Applicant’s drainage plans. Moreover, the DRB should also consider any public comments made and elicit any further public comments not already made.

- Recommended Conclusion: Unless more information is required to proof satisfaction with the Subdivision drainage standards, APPORVE the application as presented.

See https://www.richmondvt.gov/calendar/meeting/development-review-board-6-10-26.

RZR § 5.4.

RSR §§ 140.3-140.4.

RSR § 704.

RSR § 708.

RSR § 130.

RSR § 705.

RZR § 8.2.5(e).

RSR § 420.2.

RSR § 420.3.

RSR § 320.

RSR §§ 600-680.

RSR § 420.

RSR § 420.1.

RSR § 650.1.

RSR § 650.2.

RSR § 650.3.

RSR § 650.5.

RZR § 4.10.2(f).

See Vermont Urban and Community Forestry Program, Vermont Tree Selection Guide (2nd Edition), at p. 16 and 24, https://vtcommunityforestry.org/sites/default/files/2022-11/complete-vt-tree-selection-guide-2022.pdf.

See RSR §640.1.

RSR § 500(3).

RSR § 500(4).