Stormwater and EPSC management  - draft #1 of proposed  section 6.16     3.12.

6.16 Erosion Prevention and Sediment Control (EPSC), and Stormwater Management

6.16.1  Purpose:   This section is designed to address the growing problem of  increased rainfall combined with higher density of housing, with less open ground to absorb the water.  There are several goals.  The goal of erosion prevention and sediment control is to protect water quality.  Soil and pollutants are easily eroded from disturbed construction sites and end up in streams and other surface waters.  The goals of stormwater management are to prevent flooding, to prevent stream bank erosion, and to protect downstream structures and properties from damage.  After construction sites are stabilized to avoid run-off,  systems need to be installed and maintained for long-term management of  precipitation.

Thus,  there are 2 aspects to this section:

1) short-term prevention of run-off during construction when the land is disturbed,  and

2) long-term management of systems designed to control stormwater and run-off in

completed projects

6.16.2  Applicability:   All development in all districts which involve one-time or phased construction projects that disturb between 200 square feet and  1 acre of land  (small-scale development)  will be required to adhere to the standards of this section.  Projects which involve greater than 1 acre of total land disturbance must also meet these requirements;  in addition,  a state Stormwater Discharge permit will be required, along with a professionally-prepared EPSC plan and municipal DRB review.   Projects creating 3 acres or more of impervious surfaces will be required to adhere to the state’s 3-acre stormwater rule.  Applications for municipal  zoning permits  will include a checklist that shows how each applicant will meet the standards described in this section.

6.16.3  Project stages :

There are three  stages in a land development project that require action in order to carry out the purposes of this section.  The  are:  Pre-construction (planning), Construction (erosion prevention and sediment control), and Post-construction (stormwater management).

The requirements, standards and guidelines for each stage are covered in sections 6.16.4, 6.16.5 and  6.16.6 below.  There are two texts that will be used as basic references to provide guidance for best management (BMP) practices :

Low Risk Site Handbook for EPSC      (construction phase); and

Vermont Low Impact Development Guide for Residential and Small

Sites    (stormwater management phase))

Additional references that may be useful include:

State Stormwater Manual and Design Guidance (for state-permitted projects)

State Green Infrastructure (website)

Green Stormwater Infrastructure Guide for Homeowners

Vermont  Department of Environmental Conservation / Stormwater Program

6.16.4   Pre- Construction Requirements,  Standards and Guidelines

For  all  proposed construction sites, plans must be made to control all sources of sedimentation and erosion during construction, and  to protect the existing natural site features that will aid in stormwater management for the completed project.   These plans should reference activities  described in the Low Risk Handbook and the Low Impact Development Gude.  The applicant should be able to explain how each of the standards will be addressed.

a) Requirements

i) Provide a minor or major site plan as described in section 5.5.  This plan

must include all  items in order to comply with this section.

ii) Contact the Agency of Natural Resources through the Permit Navigator to

locate natural resource information for the site plan, and to seek state

permitting  if needed

iii) Engage Dig Safe to locate underground utilities

iv)  Examine the parcel deed to establish boundaries and other information

v)   Provide the construction site personnel with a copy of the site plan.

b) Standards

i) Land disturbance shall be minimized. Development of a lot or site shall

require the least amount of vegetation clearing, soil disturbance, duration

of exposure, soil compaction and topographic changes as possible.

ii) To the extent feasible, soils best suited for infiltration shall be retained and

natural areas consisting of tree canopy and other vegetation shall be

preserved, preferably in contiguous blocks or linear corridors.

iii) The time the soil is left disturbed shall be minimized. The ZAO or DRB may

require project phasing to minimize the extent of soil disturbance and

erosion during each phase of site development.

iv) There shall be no soil compaction except in the construction disturbance

area, which shall be identified and delineated in the field with appropriate

safety or landscape fencing. In areas outside the disturbance area there

shall also be no storage of construction vehicles, construction materials, or

fill, nor shall these areas be used for circulation.

v)  Development on steep slopes equal to or in excess of 15%, or which

results in such slopes, shall be subject to the provisions of Section ______.

vi) Mature trees intended to be retained shall be indicated on the plan for

protection during construction.

vii) Natural areas shall be preserved when feasible.  Development shall not

result in an undue adverse impact on fragile environments, including

wetlands, wildlife habitats, streams, lakes, steep slopes, floodplains and

vegetated riparian buffers.

viii) Open space and natural resource protection areas shall be retained

preferably in contiguous blocks or linear corridors where feasible; and the

best stormwater management features shall be identified.

ix)  Forested lands located on stream and wetland buffers and steep slopes

are priority areas and shall be preserved (see Sections _____).

x) Lot coverage and building footprints shall be minimized and where

feasible, development clustered to minimize site disturbance and preserve

large areas of undisturbed space. Environmentally sensitive areas, such as

steep slopes  shall be a priority for preservation and open space.

xi) A state permit is required if the project will disturb more than 1 acre of

land.

c)   Guidelines and BMPs

i)  Information about the preparation of a site plan is found in section 5.5 of

these regulations.

ii) Methods of planning for the standards listed above can be found in the

Low Risk Handbook and other references from the Stormwater Division

Of the Department of Environmental Conservation.

6.16.5  Construction Requirements, Standards and Guidelines

For all active construction sites,  erosion prevention and sediment avoidance shall be required as described below, following the guidelines of the BMPs  from the Low Risk Handbook.  It is essential that ESPC measures be installed prior to the commencement of construction.

a) Requirements

i)   Physically mark the limits of construction and proposed area of land

disturbance.

ii)  Create a stabilized construction access.

III) Determine locations and extent of proposed stockpiles.

iv)  Install perimeter measures to keep all runoff and sediment on-site.

v)    For extensive runoff that cannot be kept on-site, control erosion and

sedimentation by diverting runoff;  providing a conveyance for the runoff,

or dispersing concentrated flows.

vi)  Protect storm drain inlets.

vii) Provide temporary and final soil stabilization within 14 days of initial

disturbance.

viii) Create a plan for construction site inspection, maintenance and record-

keeping.

ix)  Post a notice of authorization prominently at all entrances to the

construction site.

x)  ) It is critical to prevent any runoff during  the construction phase from

impacting permanent stormwater management facilities before the soil

has been completely stabilized

b)  Standards

i)   Runoff from above the construction site must be intercepted and directed

around the disturbed area by a diversion berm.

ii)  On the site itself water must be controlled and kept at low velocities to

reduce erosion in drainage channels.  Silt fencing, erosion control berms,

filter socks or straw wattles may be used.

iii)  The amount of sediment produced from areas of disturbed soils shall be

minimized by utilizing control measures such as vegetated strips,

diversion dikes and swales, sediment traps and basins, check dams,

stabilized construction entrances, dust control, and silt fences.

iv) Mature trees intended to be retained shall be protected  by

fencing, or other appropriate barrier, placed around the tree at least 1.0 –

1.5 times the diameter of the dripline of the tree; and shall also be

protected from  below- ground disturbance by underground utility lines or

pipes.

v)   Immediate seeding and mulching or the application of sod or erosion

control matting shall be completed at the conclusion of each phase of

construction, or at the conclusion of construction if not phased.

vi)  For any construction that occurs between October 15th and April 15th,  the

applicant shall follow the erosion prevention and sediment control

practices  as outlined in Winter Construction Limitations  in The

Vermont Standards and Specifications for Erosion Prevention and

Sediment Control or the most recent Agency of Natural Resources

standards for winter construction.

c) Guidelines and BMPs

i)  Consult the Low Risk Handbook and the Watershed Management

Division of the Vermont Department of Environmental Conservation for

guidelines and a comprehensive discussion of techniques.

6.16.6   Post-Construction Stormwater Management, Standards and Guidelines

All projects are required to plan for the post construction, long-term management of stormwater to ensure that acceptable drainage arrangements and infiltration are maintained to the maximum extent practical.  Low Impact Design  (LID) principles incorporating soil characteristics, slope and other relevant characteristics should be used to guide strategies for these plans.

To the extent that LID design approaches are not included in the stormwater management plan, the applicant shall provide a full justification and demonstrate why the use of LID approaches is not possible before proposing to use conventional structural stormwater management measures which channel stormwater away from the development site.

a) Requirements

i)  Design  stormwater management systems to prevent runoff from  critical

storm events and precipitation volumes (see Standards below)

ii)  Consider amount and location  of impervious surfaces, as well as

ratio of impervious to pervious surfaces on the parcel (see

Standards below).

iii)  Design a system that aligns with existing or future neighboring

stormwater  infrastructure.

iv)  Use techniques that can be maintained  with the least cost and effort by

the intended owners, users or managers of the  parcel,  but which will

still achieve the purposes of this section

v)     Use techniques  that will function indefinitely into the future with proper

care and maintenance.

b) Standards

i)  The stormwater system  should be capable of managing the rainfall from

either  a 24-hour rain event or  a 5-year storm event given the parameters

of the site such as slope, soil types,  existing vegetation and other

factors.

ii)  The system should  consider the amount, location and ratio  of

impervious to pervious surface created by the project.  A state permit is

also  required if more than ½ acre of impervious surface is created.

c) Guidelines

There are a variety of means by which the applicant can achieve long-term stormwater management. The following guidelines should be used to assist applicants and administrators in developing maintenance plans,   Other options for complying with the purposes of this section may be found in the Low Impact Development Guide.

a) Vegetation and Landscaping. Vegetative and landscaping controls that intercept the path of surface runoff shall be considered as a component of the comprehensive stormwater management plan.

i) Design parking lot landscaping to function as part of the development’s

stormwater management system utilizing vegetated islands with bioretention

functions.

ii) Incorporate existing natural drainage ways and vegetated channels, rather than

the standard concrete curb and gutter configuration to decrease flow velocity and

allow for stormwater infiltration.

iii) Divert water from downspouts away from driveway surfaces and into bioretention

areas or rain gardens to capture, store, and infiltrate stormwater on-site.

iv) When necessary, construct vegetative LID stormwater controls (bioretention,

swales, filter strips, buffers) on land held in common.

b) Development on Steep Slopes. Development on steep slopes equal to or in excess of 15% shall be sited and constructed, and slopes stabilized to minimize risks to surface and ground waters and to protect neighboring properties from damage. With the exception of land development associated with the operation, maintenance and expansion of an alpine ski area, development shall not take place on slope gradients of 35% or more. All development on slopes equal or in excess of 15% is subject to Conditional Use Review under Section _____.

i) Locate house sites, subsurface sewage systems and parking areas on the flattest

portion of the site.

ii) Minimize crossing steep slopes with roads and driveways and lay them out to

follow topographic contours in order to minimize soil and vegetation disturbance.

Avoid long driveways.

c) Reduce Impervious Surfaces. Stormwater shall be managed through land development

strategies that emphasize the reduction of impervious surface areas such as streets, sidewalks, driveway and parking areas and roofs.

i) Evaluate the minimum widths of all streets and driveways to demonstrate that the

proposed width is the narrowest possible necessary to conform to safety and

traffic concerns and requirements.

ii) Reduce the total length of residential streets by examining alternative street

layouts to determine the best option for increasing the number of homes per unit

length.

iii) Reduce driveway lengths by minimizing setback distances. Encourage common

driveways.

iv) Use permeable pavement for parking stalls and spillover parking, sidewalks,

driveways and bike trails.

d)  Low Impact Integrated Management Practices (IMPs). Stormwater should be managed through  the use of small-scale controls to capture, store and infiltrate stormwater close to its source.

i) Create vegetated depressions, commonly known as bioretention areas or rain

gardens that collect runoff and allow for short-term ponding and slow infiltration.

Rain gardens consist of a relatively small depressed or bowl-shaped planting bed

that treats runoff from storms of one inch or less.

ii) Locate dry wells consisting of gravel or stone-filled pits to catch water from roof

downspouts or paved areas.

iii) Use filter strips or bands of dense vegetation planted immediately downstream of

a runoff source to filter runoff before it enters a receiving structure or water body.

Natural or man- made vegetated riparian buffers adjacent to waterbodies

provide erosion control, sediment filtering and habitat.

iv) Utilize shallow grass-lined channels to convey and store runoff.

v) When paving, use permeable paving and sidewalk construction materials that

allow stormwater to seep through into the ground.

vi) Consider other LID techniques such as rooftop gardens and/or rain barrels and

cisterns of various sizes that store runoff conveyed through building downspouts.

Rain barrels are generally smaller structures, located above ground. Cisterns are

larger, often buried underground, and may be connected to the building’s

plumbing or irrigation system.

vii) Add minerals and organic materials to soils to increase its capacity for absorbing

moisture and sustaining vegetation.